URBEM UPGRADING PLAN – DESCRIPTION OF TASK AND PLAN

Workstream A: Sector-Specific and Pollutant-Specific Proxy Weighting

- Data Compilation and Weight Derivation

Objective

Derive scientifically grounded per-pollutant, per-CORINE-class weights for major emitting sectors (agriculture, waste, industry, residential).

| Task | Completed |
| --- | --- |
| Agriculture poxy weights | TO DO |
| Industrial and Waste Proxy weights | TO DO |
| Residential proxy weights | TO DO |
| Documentation and Validation | TO DO |

- Multi-Proxy Blending Configuration (All Sectors)

Objective
Extend your existing road-type weighting infrastructure to all sectors, creating a consistent multi-proxy framework.

| Task | Completed |
| --- | --- |
| Multi-Proxy Blending Framework Architecture | TO DO |
| Data Sources for Multi-Proxy Components | TO DO |

- Ensemble-Based Conflict Resolution and Uncertainty Quantification

Objective

Implement the three-option ensemble conflict resolution approach and quantify model uncertainty through systematic parameter perturbation.

| Task | Completed |
| --- | --- |
| Ensemble Configuration Framework | TO DO |
| Ensemble Downscaling Execution | TO DO |
| Uncertainty Interpretation and Visualization | TO DO |
| Literature Documentation | TO DO |

Workstream B: Sectoral Enhancements

- Agriculture Livestock–Crop Differentiation

Objective

Separate agricultural emissions into crop-driven (N2O) and livestock-driven (CH4) components with distinct spatial proxies.

| Task | Completed |
| --- | --- |
| Livestock Density Grid Development | TO DO |
| Fertilizer Application Proxy | TO DO |
| Multi-Proxy Aggregation for Agriculture | TO DO |

- Waste Hybrid Point-and-Area Downscaling

Objective

Replace the uniform waste proxy with a hybrid approach treating major landfills and incinerators as point sources and diffuse waste handling as area sources.

| Task | Completed |
| --- | --- |
| Point-Source Facility Identification and Emission Estimation | TO DO |
| Area-Source Fallback and Blending | TO DO |
| Waste-Sector-Specific Pollutants | TO DO |

- Integration, Testing, and Documentation

Objective

Merge Workstream A (proxy weighting, multi-proxy blending, ensemble uncertainty) with Workstream B (agriculture, waste) into a unified, tested system.

| Task | Completed |
| --- | --- |
| Full Pipeline Integration and Testing | TO DO |
| Sensitivity Analysis and Robustness Documentation | TO DO |
| User Guide and Configuration Examples | TO DO |

Task 1.1: Agriculture Proxy Weights (Livestock vs. Crop Differentiation)

Scientific Literature Foundation: The EMEP/EEA air pollutant emission inventory guidebook 2023  and EPA's agricultural emissions methodology provide emission factors by crop type and livestock category. Additionally, recent work on livestock methane proxies details how different animal types contribute differently to methane emissions based on herd composition and feeding practices.

Implementation : create a weighting matrix for livestock and crop weights

Derivation Method: These weights are calculated from EMEP/EEA emission factor ratios. For example, EPA data16 shows that dairy cattle CH4 emissions are approximately 150 kg CH4/head/year, while arable land N2O emissions average ~0.3 kg N2O/ha/year under typical European fertilizer application (150 kg N/ha/year × 0.01 kg N2O-N/kg N). Normalizing across CORINE class areas in a sample European domain, pasture-dominated regions have ~1.5× higher per-hectare livestock methane intensity than average agricultural land. These ratios become your weights.

Data sources : 

MEP/EEA Guidebook 2023 (Agriculture)

https://www.eea.europa.eu/en/analysis/publications/emep-eea-guidebook-2023

EPA 2020 National Emissions Inventory (Agriculture)

https://www.epa.gov/system/files/documents/2023-03/NEI2020_TSD_Section16_AgCrops_LivestockDust.pdf

Methane livestock specific mitigation :
https://pubmed.ncbi.nlm.nih.gov/41455311/

Task 1.2: Industrial and Waste Proxy Weights

Scientific Literature Foundation: CAMS-REG-v4 inventory18 provides facility-level point-source emissions and area-source spatial patterns for industrial sectors. The Nordic air pollution inventory9 9 discusses how E-PRTR facility density varies by industrial CORINE class and SNAP subcategory. Recent work on industrial emissions spatial distribution21 quantifies how different industrial classes (chemical, metal smelting, petroleum refining, paper manufacturing) concentrate in distinct CORINE polygons.

Implementation :

For Industrial GNFR sectors weight CORINE industrial classes by they emission intensity relative to mean industrial emissions.

- CORINE 121 (industrial/commercial units): Average case; weight = 1.0 (baseline)

- CORINE 122 (road/rail networks in industrial areas): Lower intensity; weight = 0.4 (infrastructure, not combustion)

- CORINE 123 (port areas): High intensity due to port equipment; weight = 1.8

- CORINE 124 (airports): Moderate-to-high intensity; weight = 1.2

For waste (GNFR E + F, combined into a single waste proxy in your current config), differentiate:

- Landfill sites : CORINE 211 & 121 - Landfills emit CH4; incinerators emit NOx

- Waste/recycling facilities : 121 - Incinerators/recycling typically in industrial zones

- Sewage/wastewater - WWTP near water; primarily NOx from aeration

- Create appropriate weights for each facility type

Data Sources :

CAMS-REG-v4 inventory

https://essd.copernicus.org/articles/14/491/2022/

MEP/EEA Guidebook 2023 (waste)

EPA greenhouse gas inventory (waste section)

Landfill methane estimation methods

Task 1.3: Residential Combustion Proxy Weights

Scientific Literature Foundation : Recent high-resolution mapping of residential wood-burning emissions30 shows substantial heterogeneity in heating sources by property type and region. Building-level energy performance certificates (EPCs) reveal that wood burners concentrate in rural and suburban areas but exhibit different densities by building type (detached houses, apartments, etc.). This work provides empirical evidence that a uniform "residential" proxy underestimates emissions in wood-burning-intensive regions.

Implementation : For residential combustion (GNFR D) create CORINE class weitghs reflecting heating source distribution :

- 111 = Continuous urban fabric

- 112 = Discontinuous urban fabric

- 122 = Industrial/commercial (residential above)

- 141 = Green urban areas

- 142 = Sport/leisure facilities

Additional improvements : For regions with significant wood burning (much of Northern/Central Europe), introduce a climate-adjusted weight based on heating-degree-days (HDD). EPC-based research30 shows that regions with HDD > 3500 (cold climates) exhibit 2–3× higher wood-burner prevalence than temperate regions. If available, apply a HDD scaling factor:

Wadjusted= Wbase ×(1+0.0001×max0,HDD-2000)

This factor starts at 1.0 for HDD = 2000 and increases ~10% per 1000 HDD above that threshold.

Data source

High-resolution mapping of residential wood-burning emissions

https://pmc.ncbi.nlm.nih.gov/articles/PMC7618568/

EMEP/EEA Guidebook (residential combustion)

National heating appliance surveys or EPC databases

Task 1.4: Documentation and Validation Script

Create a configuration file documenting all weights, their source references, and uncertainty ranges. Example structure:

```

{ "weights_metadata":

{ "agriculture_K_livestock":

{ "description": "CORINE class weights for GNFR K (livestock emissions)",
"source": "EMEP/EEA Guidebook 2023 Appendix A, EPA NEI 2020 Section 16",
reference_year": 2020, 
"uncertainty_low": 0.85, 
"uncertainty_high": 1.15, 
"notes": "Weights derived from emission factor ratios; ±15% range reflects livestock census variability" }

} }

```

Develop a validation script that:

- Loads these weights

- Recalculates aggregated proxies with weighting

- Compares weighted vs. unweighted proxy spatial patterns

- Exports visualization showing where weights change proxy distributions most

This produces concrete evidence of the upgrade's impact before full deployment.

Task 2.1: Multi-Proxy Blending Framework Architecture

Scientific Literature Foundation: The CAMS-REG-v4 inventory and Nordic air pollution inventory both use multiple proxies per sector, blending road networks with traffic intensity, population with urban-centre indicators, and land use with facility data. Your existing code already supports road-type weighting for line emissions; this task generalizes that capability to all sectors.

Implementation: Create a configuration structure allowing each GNFR sector to specify multiple component proxies and blend weights:

Exemple :

```

"D_residential":

{ "components": [

{ "name": "building_density", 
"data_source": "ghsl_built_volume", 
"weight": 0.4 
}, 
{ "name": "population_density", 
"data_source": "ghsl_population", 
"weight": 0.35 
}, 
{ "name": "heating_intensity", 
"data_source":
 "heating_degree_days",
 "weight": 0.25
 } ],

"combination_method": "weighted_sum" 
},

```

The computation becomes :

Pcompositei,j= cwc*Pc(i,j)cwc

Where wc is the component weight, Pc(i,j) is the normalized component proxy at cell (i,j) and all components are normalized to 0-1 before blending.

Task 2.2: Data Sources for Multi-Proxy Components

| Sector | Component 1 | Component 2 | Component 3 | Source |
| --- | --- | --- | --- | --- |
| Road Transport | OSM road network (type weights) | Population density | Urban-centre indicator GHSL | High resolution spatial parameters for road transport and off-road sector OSM in environmental exposure |
| Residential | Building volume - GHSL | Population - GHSL | Heating degree days | Copernicus |
| Agriculture | Corine Land cover | Livestock density grid |  | Methane proxies Emissions inventory from US |
| Waste | Corine 121/211 + E-RPRT enhanced | Point sources (landfills/incinerators) | Proximity to urban centers | EPA – waste chapter |
| Shipping | Port locations (Corine) | Shipping lanes | Ship traffic patterns AIS data |  |

Implementation : OSM, CORINE and E-RPRT is already handled. We need to integrate livestock density grids and heating degree days

Code Changes: Modify your downscaling loop to:

- Load multi-proxy configuration for the current GNFR

- Retrieve each component raster

- Normalize each component to 0–1 independently

- Compute weighted sum per cell

- Normalize final composite to 0–1 across the domain

- Multiply by CAMS totals as before

Task 3.1: Ensemble Configuration Framework

Scientific Literature Foundation: Recent work on uncertainty quantification in transportation emissions and Bayesian approaches to prediction intervals demonstrates the value of ensemble methods for capturing both epistemic (model structure) and aleatoric (data) uncertainty. Your approach will implement a lightweight ensemble by running downscaling multiple times with systematically perturbed parameters.

Implementation:

Define an ensemble configuration specifying which parameters to vary and their perturbation ranges:
Example
```

{

"parameter": "corine_class_weights", 
"sectors": ["agriculture", "industry", "waste"], 
"perturbation_method": "gaussian", 
"perturbation_sigma": 0.15, 
"bounds": [0.7, 1.3]

}

```

Perturbation Strategies:

- Gaussian perturbation for CORINE class weights: Each weight is drawn from , clipped to [0.7, 1.3] to prevent unrealistic values. This reflects ±15% uncertainty in emission factor ratios.

- Dirichlet perturbation for multi-proxy component weights: For a sector with three components (e.g., road, population, urban), draw a sample from a symmetric Dirichlet distribution with concentration parameter α = 10. This produces weight triplets (w1, w2, w3) centered on (0.4, 0.35, 0.25) but with natural variation reflecting uncertainty in optimal blending.

- E-PRTR facility-confidence perturbation: Vary each facility's confidence score by ±10%, affecting how it blends with CORINE fallback proxies.

- Conflict-resolution rotation: Systematically cycle through three conflict strategies (hierarchy, soft blending, equal) across ensemble members. For example, members 1–33 use hierarchy, 34–66 use soft blending, 67–100 use equal contribution.

Task 3.2: Ensemble Configuration Framework

Code Changes:

Create an ensemble runner that:

- Reads ensemble configuration

- For each realization (1 to N):

  - Perturb selected parameters according to configuration

  - If rotation-based conflict resolution, select strategy for this realization

  - Execute full downscaling pipeline with perturbed parameters

  - Save gridded result to temporary storage

- Once all realizations complete, compute statistics:

  - Ensemble mean per cell:
 μi,j=1N⋅n=1NEni,j

  - Ensemble standard deviation:
 σi,j=1N⋅n=1N(Eni,j-μi,j2

  - Percentiles (5th, 25th, 50th, 75th, 95th)

  - Export mean, standard deviation, and percentile rasters

If possible

Parallelization: This is embarrassingly parallel; run each realization on a separate CPU core. With 100 realizations and a single downscaling run taking ~5–10 minutes on 1 km² grid, a 16-core machine completes the ensemble in ~30–60 minutes.

Task 3.3: Uncertainty Interpretation and Visualization

Deliverables:

- Ensemble mean grid: Your best estimate of downscaled emissions

- Uncertainty grid: Standard deviation raster showing confidence (low std = robust downscaling; high std = sensitive to parameter choice)

- Percentile grids: 5th and 95th percentiles define a ~90% plausible range for emissions in each cell

- Diagnostic plots:

  - Scatter plot of mean vs. std by grid cell, identifying high-uncertainty regions

  - Spatial map of coefficient of variation (std / mean) highlighting sectors/regions most affected by parameter uncertainty

  - Histogram of ensemble member totals per GNFR sector, showing distribution of total sectoral emissions across realizations

Use Case: If a city's policy decision hinges on emissions in a specific cell or sector, these diagnostics reveal whether the model output is robust or uncertain. For example, if industrial emissions have high ensemble std in a specific location, that suggests your proxy choice (E-PRTR vs. CORINE) matters significantly, warranting targeted data collection or expert review.

Task 3.4: Literature Documentation

Create a brief technical note documenting your ensemble approach, citing methodology papers on uncertainty quantification in environmental modeling

TASK 4.1: Livestock Density Grid Development

Scientific Literature Foundation: Recent work using satellite imagery to detect livestock manure spreading26 and methane proxy studies7 demonstrate the feasibility of deriving livestock-density proxies from remote sensing and agricultural census data. The EPA agricultural emissions methodology16 provides emission factors by livestock type.

Implementation:

- Data Source: Obtain livestock census data from Eurostat (if EU domain) or national agricultural ministries (livestock counts by municipality/region, disaggregated by cattle, pigs, poultry, sheep).

- Interpolation: Assuming your study domain has livestock census data at the regional level (e.g., NUTS3 regions or counties), use areal interpolation to generate a fine-resolution livestock-density grid:

  - Restrict the animal population to CORINE pasture and permanent crop classes (classes 13, 18–20)

  - Distribute animals proportionally to pasture area within each census region

  - Produce a raster of livestock units per hectare at your target resolution (e.g., 1 km)

- Livestock Unit Standardization: Convert counts to standardized livestock units (LSU) where 1 LSU ≈ 1 dairy cow. Use conversion factors:

  - Beef cattle: 0.6 LSU/head

  - Dairy cattle: 1.0 LSU/head

  - Pigs: 0.2 LSU/head

  - Poultry: 0.01 LSU/head

  - Sheep: 0.15 LSU/head

- Emission-Factor Weighting: Weight each livestock type by its methane emission intensity from EPA/EMEP-EEA data :

Normalized by mean intensity, dairy cattle receive weight ~2.5× pigs. Apply these weights to livestock density before normalization.

Data Source :

EPA/EMEP-EEA

TASK 4.2: Fertilizer Application Proxy (Crop N2O)

Scientific Literature Foundation: European fertilizer sales data and agricultural management practices reveal that nitrogen application concentrates in arable and permanent-crop regions9 . Recent Nordic inventory work9 uses regional fertilizer application rates combined with CORINE crop classes to spatialize N2O emissions.

Implementation:

- Regional Fertilizer Data: Obtain annual fertilizer application rates (kg N/ha/year) by crop type and region from Eurostat, national agricultural agencies, or the FAO. If available, use the JRC CAPRI model output which provides detailed spatially distributed fertilizer application.

- Crop-Specific Allocation: For each CORINE arable/permanent-crop class, assign a typical nitrogen application rate:

  - Arable land (class 12): 150 kg N/ha/year (typical wheat, corn)

  - Permanently irrigated (class 13): 200 kg N/ha/year (higher intensity)

  - Rice fields (class 14): 120 kg N/ha/year

  - Permanent crops (class 15): 80 kg N/ha/year (vineyards, orchards, lower N needs)

  - Complex cultivation (class 20): 120 kg N/ha/year (mixed)

- N2O Emission Intensity: Apply crop-specific N2O emission factors from EMEP-EEA :

  - Typical direct N2O from applied N: ~0.01 kg N2O-N / kg N applied

  - Pasture (class 18): ~0.02 kg N2O-N / kg N (animal waste)

Compute a per-pixel N application rate (kg N/ha), convert to N2O using factors, producing a crop N2O proxy raster

Data sources : 
https://gmd.copernicus.org/preprints/gmd-2021-390/gmd-2021-390-manuscript-version3.pdf

https://www.eea.europa.eu/en/analysis/publications/emep-eea-guidebook-2023

TASK 4.3: Multi-Proxy Aggregation for Agriculture

Reconfigure your GNFR K (livestock) and L (other agriculture) downscaling:

Exemple

Livestock : 80% livestock density, 20% Corine pasture

AgriOther : fertilizer : 0.6, 40% arable Corine

TASK 4 Deliverables:

- Livestock-density grid (LSU/ha) for your domain

- Fertilizer-application proxy (kg N/ha) by crop type

- Revised GNFR K and L downscaled grids

- Comparison visualization: original unified agriculture proxy vs. differentiated livestock/crop proxies

TASK 5.1: Point-Source Facility Identification and Emission Estimation

Scientific Literature Foundation: Landfill methane estimation methodologies17 8 provide technical guidance on facility-level emissions; CAMS-REG-v418 separates point and area sources for waste; recent satellite monitoring of landfills26 demonstrates feasibility of facility location and activity assessment.

Implementation:

- Facility Database: Compile a list of major landfills and incinerators in your study domain. Data sources:

  - E-PRTR: Filter for NACE codes 38.1 (waste collection), 38.2 (waste treatment/disposal), 38.3 (recovery)

  - National waste registries: Many EU member states maintain landfill and incinerator registers

  - Open data portals: EU waste facilities databases

- Facility-Level Emissions:

  - Landfills: Use E-PRTR reported CH4 emissions if available. If unavailable, estimate from first-order-decay (FOD) model using landfill size (m³), waste type, and operational status8 17 .

  - Incinerators: Use E-PRTR reported NOx, SO2, and particulate emissions. If unavailable, estimate from capacity and fuel composition8 .

- Point-Source Representation: Create a point-source raster where:

  - Each facility location receives its estimated annual emissions

  - Spatially smooth over a small radius (e.g., 500 m) to account for plume dispersion and location uncertainty

  - Generate separate rasters for CH4 (landfill-dominated) and NOx (incinerator-dominated)

Data sources :

Using Ambient Methane Concentrations to Estimate Total Landfill Emissions

https://www.hinkleycenter.org/pdfs/2022-02_Abichou.pdf

EPA (waste chapter)

CAMS REG

TASK 5.2: Area-Source Fallback and Blending

Reconfigure waste downscaling as : 
landfills : emission : CH4

Incinerators  : emissions : Nox, SO2

Area sources : Corine 211, 121+  eptr

Workflow:

- Calculate GNFR E (waste) total emissions from CAMS for your domain

- Subtract point-source (landfill + incinerator) emissions from the total

- Distribute the remainder (diffuse waste handling, small facilities) using your area-source proxy (CORINE 211/121 + E-PRTR flags)

- Combine point and area outputs

TASK 5.3: Waste-Sector-Specific Pollutants

Literature Basis: EPA waste sector guidance8 and landfill methane studies17 show that methane dominates landfill emissions while NOx and particulates dominate incinerators. Your downscaling should reflect this. Example :

```

{ "waste_pollutant_routing": 
 	{ "CH4": { "landfill_fraction": 0.95, "incinerator_fraction": 0.05 },

"NOx": { "landfill_fraction": 0.05, "incinerator_fraction": 0.95 },

"PM10": { "landfill_fraction": 0.1, "incinerator_fraction": 0.9 } }

}

```

TASK 5 Deliverables:

- Landfill and incinerator facility database with location and emissions estimates

- Point-source rasters (CH4 for landfills, NOx for incinerators)

- Revised waste proxy combining point + area sources

- Comparison visualization showing spatial shift from uniform proxy to facility-concentrated distribution

Task 6.1: Full Pipeline Integration and Testing

Objective: Merge Workstream A (proxy weighting, multi-proxy blending, ensemble uncertainty) with Workstream B (agriculture, waste) into a unified, tested system.

Implementation:

- Configuration Consolidation: Merge all JSON configuration files (weights, multi-proxy, ensemble, sectoral) into a master configuration or organized subdirectory structure.

- Integration Testing: Run full downscaling on 2–3 test cities with all upgrades enabled. Validate:

  - Weighted proxies aggregate correctly (sum to 1.0 per domain)

  - Multi-proxy blending produces valid spatial distributions

  - Ensemble runs complete without error; mean/std/percentile rasters are valid

  - Sectoral proxies (agriculture K/L, waste) show expected spatial differentiation

- Performance Profiling: Measure runtime impact. Ensemble 100× realizations should add ~50% total runtime (parallelizable). Multi-proxy blending and sectoral enhancements should add <20%.

Task 6.2: Sensitivity Analysis and Robustness Documentation

Deliverables:

- Sensitivity Report: For each major upgrade, quantify impact on total and spatial distribution of emissions:

  - How much do weighted proxies change total GNFR K and L emissions relative to uniform weights?

  - How much spatial variance is explained by multi-proxy blending vs. uniform population?

  - How wide are ensemble confidence intervals (% of mean emissions)?

  - How much does sectoral separation (livestock vs. crop) reallocate emissions spatially?

- Uncertainty Quantification Summary:

  - Per-sector ensemble coefficient of variation (std / mean)

  - Spatial maps of high-uncertainty regions

  - Recommendations on which sectors/regions require additional data refinement

- Scientific Documentation: Write a technical note explaining methodology choices, citing all literature sources14 15 16 18 21 9 7 8 17 30 33 . Include:

  - Equations for weighted aggregation and multi-proxy blending

  - Description of ensemble perturbation strategy

  - Justification for sectoral proxy choices

  - Known limitations and future improvements

Task B3.3: User Guide and Configuration Examples

Deliverables:

- Configuration Template: Provide a well-commented JSON file showing all available options for weights, proxies, and ensemble settings.

- City-Specific Calibration Guide: Document how users should customize configurations for their domain:

  - Which CORINE class weights are region-specific (e.g., livestock vs. crop intensity varies by agroclimatic zone)?

  - Where are reference data (livestock census, fertilizer sales, facility registers) obtained?

  - How should users adapt multi-proxy weights if their domain has atypical characteristics (e.g., heavy port traffic, wood burning)?

- Validation Checklist: Provide a checklist for users to verify their downscaled results:

  - Do total emissions match CAMS inputs? (Should be exact within rounding)

  - Do spatial patterns align with domain knowledge? (E.g., industry concentrated near known industrial zones?)

  - Are ensemble confidence intervals reasonable? (Typically 10–30% of mean for well-constrained proxies?)

Implementation : 

Task 1.1 -> dynamic weighting across Corine class and pollutant for NUTS – 2 Regions

CH4 :

1) For each NUTS2 polygon, read Eurostat animal counts (thousand heads) from the

six sheets above; convert to heads; compute a scalar CH4_proxy for the region as

the sum over categories of (heads x category EF).

2) Overlay the CORINE raster (CLC level-3, nodata excluded) on the same NUTS2

boundaries; count pixels per CLC inside each region.

3) Restrict to agricultural CLC codes 12-22. Within each NUTS2, split the regional

CH4_proxy across those CLCs in proportion to each class's share of agricultural

pixels in that region (same proportion as agricultural area at this resolution).

4) CH4_WEIGHTS is that split renormalized so that, for a given NUTS_ID, the eleven

rows (CLC 12..22) sum to 1. If there is no regional CH4_proxy > 0 or no ag pixels,

CH4_WEIGHTS is left blank (missing) for that region's rows.

Dynamic weighting for Nox :

Agricultural Nox comes from 3 sources : soil nitrification, manure application, machinery combustion